# AirOne V7.1 — Connecting a Real CanSat (LoRa E22 over USB)

This guide explains how to receive live telemetry from a physical CanSat on your laptop using a LoRa E22 module connected via a USB-to-serial bridge. It covers wiring, E22 configuration, launching the ground station in live mode, and troubleshooting.

Scientific-honesty guarantee. The ground station never fabricates telemetry. If no radio is found it reports an explicit NOT_CONFIGURED state and streams nothing. Simulation (--simulate) is an isolated testing/training mode only; real telemetry is always authoritative and simulated packets are tagged SIMULATED.

## 1. Signal chain overview

[CanSat: ESP32 + sensors + E22 (air)]  ~~LoRa RF~~  [E22 (ground) + USB-serial]  ->  laptop USB
                                                                                        |
                                                            AirOne ground station (launcher.py)

- The flight E22 is driven by the ESP32 firmware in firmware/airone_cansat/.

- The ground E22 is wired to a USB-serial bridge (CP210x / CH340 / FTDI) and appears on your laptop as a serial port.

- Both E22 modules must share the same RF settings (frequency channel, address, air data rate) and the same UART baud as the ground station.

## 2. Ground-side wiring (E22 ↔ USB-serial bridge)

Put the E22 in transparent mode (M0 = M1 = GND). In transparent mode the E22 passes bytes through unchanged — exactly what the AirOne binary protocol needs.

| E22 pin | Connect to (USB-serial bridge) |
| --- | --- |
| VCC | 3.3 V (E22 is 3.3 V logic) |
| GND | GND |
| TXD | RXD of the bridge |
| RXD | TXD of the bridge |
| M0 | GND (transparent mode) |
| M1 | GND (transparent mode) |
| AUX | (optional) leave unconnected for basic RX |

Use a 3.3 V USB-serial bridge. Feeding 5 V into E22 RXD can damage the module.

## 3. Flight-side wiring (ESP32 ↔ E22)

Defaults used by firmware/airone_cansat/airone_cansat.ino:

| ESP32 pin | E22 pin | Notes |
| --- | --- | --- |
| GPIO17 (TX2) | RXD | ESP32 → E22 |
| GPIO16 (RX2) | TXD | E22 → ESP32 |
| 3.3 V | VCC |  |
| GND | GND, M0, M1 | transparent mode |

Sensors (I²C on SDA=21, SCL=22) and the MAX-M10S GNSS (UART1: GPIO4/GPIO2) are detailed in firmware/README.md.

## 4. E22 baud rate must match

The E22’s UART baud (module setting) and the AirOne ground-station baud must be identical. The firmware and ground station both default to 115200. If your E22 modules are configured for a different UART baud (common factory default is 9600), either reconfigure them or pass the matching --baud.

Because the E22 in transparent mode has a fixed UART baud, use --force-baud so the ground station opens the port directly at that baud instead of trying to auto-negotiate:

python launcher.py --serial-port /dev/ttyUSB0 --baud 115200 --force-baud

## 5. Finding the serial port

- Linux: usually /dev/ttyUSB0 (FTDI/CP210x) or /dev/ttyACM0. List devices: ls /dev/ttyUSB* /dev/ttyACM* or dmesg | grep tty after plugging in. You may need permission: sudo usermod -aG dialout $USER then re-login.

- macOS: /dev/tty.usbserial-XXXX or /dev/tty.SLAB_USBtoUART.

- Windows: COM3, COM5, … (see Device Manager → Ports).

Auto-discovery: if you omit --serial-port, AirOne scans for known USB-serial bridges (CP210x 0x10C4, CH340 0x1A86, FTDI 0x0403) and picks the first match.

## 6. Launching the ground station in live mode

Set the mandatory JWT secret (≥32 chars) and run the launcher:

export AIRONE_JWT_SECRET="your-long-random-secret-at-least-32-characters"

# Auto-discover the radio:
python launcher.py

# Or specify the port/baud explicitly (recommended for E22 transparent mode):
python launcher.py --serial-port /dev/ttyUSB0 --baud 115200 --force-baud

On startup the launcher prints the explicit link state, one of:

- LIVE (/dev/ttyUSB0 @ 115200 baud) — receiving live telemetry.

- NOT_CONFIGURED (no CanSat radio found …) — nothing connected; no data is fabricated. Plug in the radio (or use --simulate for isolated testing).

- DISABLED (telemetry.enabled=false) — link intentionally turned off.

The live link status is also available in the health report / API and updates bytes_received, packets_received, crc_errors, and reconnect_count.

### Configuration reference (config/default_config.yaml)

telemetry:
  enabled: true         # false = do not start the live link
  serial_port: null     # null = auto-discover; or "/dev/ttyUSB0" / "COM5"
  baud_rate: 115200     # must match the E22 UART baud
  force_baud: false     # true = open directly at baud_rate (E22 transparent)

CLI flags override config. AIRONE_SERIAL_PORT env var overrides the config serial_port (but is overridden by --serial-port).

## 7. What good telemetry looks like

Each frame the CanSat sends is CRC-protected. When frames arrive: - packets_received climbs and crc_errors stays low. - Valid frames flow through the 20-stage pipeline and appear in the GUI/API. - Corrupted frames are counted as CRC errors and discarded (never accepted with fabricated values).

## 8. Troubleshooting

| Symptom | Likely cause / fix |
| --- | --- |
| NOT_CONFIGURED but radio is plugged in | Wrong/aliased port — pass --serial-port explicitly; check ls /dev/ttyUSB*. On Linux add yourself to dialout. |
| Port opens but packets_received stays 0 | Baud mismatch — use --baud to match the E22 UART; add --force-baud. Confirm the CanSat is powered and transmitting. |
| Many crc_errors | RF interference / weak link / partial frames — check antennas, reduce distance, confirm both E22s share channel/address/air-rate. |
| Data arrives then stops | Link dropped — the receiver auto-reconnects with backoff; check power/USB. reconnect_count will increase. |
| pyserial not installed | pip install -r requirements_v71.txt (installs pyserial). |
| Bytes look garbled in a serial monitor | That’s the binary frame format — expected. Do not use a text serial monitor to validate; use the AirOne ground station. |
| Permission denied on /dev/ttyUSB0 (Linux) | sudo usermod -aG dialout $USER, then log out/in. |

## 9. Testing without hardware (isolated simulation)

When you have no radio at hand, exercise the entire real pipeline with the physics-based simulator. Simulated packets use the identical binary framing and are tagged SIMULATED:

python launcher.py --simulate --sim-loop

If a live link is also up, real telemetry remains authoritative; the launcher warns that simulation is running alongside it.