# Scientific Methods

AirOne V7.1 ships 37 deterministic scientific analyses across 9 categories, implemented in src/scientific/ and registered in src/scientific/registry.py. Every analysis returns a ScientificResult with a value, unit, explicit uncertainty, and the assumptions it relied on.

## Scientific-honesty rules

These rules are enforced in the analysis code and are non-negotiable:

- Correlation is never reported as causation. Correlation analyses (e.g. radiation_altitude_correlation, correlation_matrix) report the coefficient and sample size and explicitly label the relationship as correlation only.

- Uncertainty is always propagated. No analysis returns a point value without an uncertainty (or an explicit statement that uncertainty is not yet quantifiable, in which case the result is marked accordingly).

- Insufficient data yields an explicit “cannot compute”, never a guess. An analysis with too few valid samples returns an unavailable/insufficient result rather than a fabricated number.

- Invalid inputs are excluded, not imputed. Analyses consume only valid measurements; missing values are never zero-filled.

- Simulated inputs stay labelled. Results computed from SIMULATED data inherit that provenance.

## Categories and analyses

### Atmospheric (10)

atmospheric_density, atmospheric_stability, boundary_layer_height, dew_point, hypsometric_altitude, lapse_rate, pressure_gradient, temperature_gradient, temperature_inversion, vertical_profile.

Core relations used include the barometric/hypsometric equation for pressure-derived altitude, the environmental lapse rate (dT/dz), the Magnus/Arden-Buck formula for dew point, and the ideal-gas relation for air density. Stability and inversion detection compare the measured lapse rate to the dry adiabatic lapse rate.

### Descent (4)

descent_rate, drag_coefficient, landing_detection, terminal_velocity.

Descent rate is the time derivative of altitude; terminal velocity and the effective drag coefficient are derived from the balance of gravity and drag during steady descent. Landing detection combines low vertical speed with altitude stability.

### Gases (3)

air_quality_proxy, plume_detection, voc_profile.

VOC/gas-resistance readings from the BME688 are turned into an air-quality proxy and a vertical VOC profile; plume detection flags localised concentration anomalies.

### GNSS (4)

displacement_from_start, ground_speed, ground_track_distance, vertical_speed.

Horizontal metrics use great-circle (haversine) distance between fixes; vertical speed uses GNSS altitude deltas. All require valid GNSS fixes — with no fix, the analyses report unavailable rather than emitting zeros.

### Magnetic (4)

magnetic_anomaly, magnetic_field_magnitude, magnetic_heading, magnetic_inclination.

Field magnitude is the Euclidean norm of the three magnetometer axes; heading and inclination are derived from the field vector; anomalies are deviations from the expected local field.

### Power (3)

battery_health, brownout_risk, voltage_trend.

Voltage trend is a regression over the battery-voltage series; brownout risk compares projected voltage against a threshold; battery health summarises the discharge behaviour.

### Radiation (4)

dose_rate, mean_count_rate, radiation_altitude_correlation, radiation_burst.

Count rates (CPM) are converted to an approximate dose rate; the altitude–radiation relationship is reported strictly as a correlation with its coefficient and sample count; bursts are transient rate excursions.

### UV / Solar (2)

uv_altitude_trend, uv_index.

UV index from the UV sensor and its trend with altitude.

### Composite (3)

change_rate_monitor, correlation_matrix, environmental_hazard_index.

Cross-sensor summaries: a rate-of-change monitor, a correlation matrix across key channels (correlation only — never causal), and a composite environmental hazard index.

## Filtering and fusion

Before analyses run, the pipeline can apply configurable filters (stage 14) and sensor fusion (stage 15):

- Kalman 1-D (e.g. fused pressure), moving average (temperature), EKF altitude — all configured under pipeline.filter_config.

- Fused outputs are tagged DataSource.FUSED; filtered outputs are tagged appropriately so consumers always know the provenance of a number.

## Narrative

GET /api/v1/narrative produces a natural-language description of the mission from the computed results. The narrative is generated from real analysis outputs and states uncertainty; it never asserts findings the analyses did not support.