# Telemetry Protocol Specification

The AirOne wire format is a compact binary frame with a fixed header, a variable payload, an optional 8-byte authentication tag, and a trailing CRC32. It is implemented in src/telemetry/protocol.py.

## Frame layout

Offset  Size  Field
------  ----  -----------------------------------------------
0       4     MAGIC  = A1 60 4E 45
4       1     VERSION = 0x71
5       1     PACKET_TYPE
6       4     SEQUENCE_NUMBER   (uint32, little-endian)
10      8     TIMESTAMP         (uint64, little-endian)
18      2     PAYLOAD_LENGTH N  (uint16, little-endian)
20      1     FLAGS  (bit0=FEC_PRESENT, bit1=COMPRESSED, bit2=ENCRYPTED, bit3=AUTHENTICATED)
21      N     PAYLOAD
21+N    8     AUTH_TAG  — only when FLAGS bit3 is set
21+N+T  4     CRC32 over every preceding byte (header, payload, tag)  (uint32, LE)

PAYLOAD_LENGTH counts the payload only (never the tag). Unknown VERSION, PACKET_TYPE or FLAGS bits cause the frame to be rejected and counted (invalid_version / invalid_type / invalid_flags) — never parsed.

- Header size: 21 bytes. CRC size: 4 bytes.

- Header struct format: <4sBBIQHB.

- Maximum payload: 65535 bytes (bounded by the uint16 length field).

## Constants

| Name | Value |
| --- | --- |
| MAGIC | A1 60 4E 45 |
| VERSION | 0x71 |
| HEADER_SIZE | 21 |
| CRC_SIZE | 4 |
| MAX_PAYLOAD | 65535 |
| FLAG_FEC_PRESENT | 0x01 |
| FLAG_COMPRESSED | 0x02 |
| FLAG_ENCRYPTED | 0x04 |
| FLAG_AUTHENTICATED | 0x08 |
| AUTH_TAG_SIZE | 8 |
| MIN_LINK_KEY_BYTES | 16 |

## Packet types (PacketType)

| Value | Name |
| --- | --- |
| 0x01 | SENSOR_DATA |
| 0x02 | GPS |
| 0x03 | SYSTEM_STATUS |
| 0x04 | COMMAND |
| 0x05 | ACK |
| 0x06 | HEARTBEAT |
| 0x07 | FEC_DATA |
| 0x08 | ERROR |

## CRC32

The CRC32 is computed with binascii.crc32 masked to 32 bits and packed little-endian. It covers every byte preceding the CRC field — the full header and the payload. A frame whose recomputed CRC does not match is marked crc_valid = False; downstream, the affected data is tagged CORRUPTED and is never treated as a real reading.

## Frame authentication (FLAG_AUTHENTICATED)

AUTH_TAG = HMAC-SHA256(link_key, header ‖ payload)[0:8]. The key (≥ 16 bytes, hex) is AIRONE_LINK_KEY on the ground station and AIRONE_LINK_KEY_HEX in the firmware; compute_auth_tag() / airone_auth_tag() are byte-for-byte identical (tested against RFC 4231). The tag is compared in constant time. The resulting auth_state is explicit:

| auth_state | Frame has tag | Ground station has key | Result |
| --- | --- | --- | --- |
| NOT_CONFIGURED | no | no | accepted as before |
| AUTHENTICATED | yes | yes, matches | accepted |
| UNAUTHENTICATED | no | yes | SUSPECT; INVALID when telemetry.require_authenticated_frames is true |
| UNVERIFIABLE | yes | no | SUSPECT — cannot be checked |
| INVALID_TAG | yes | yes, mismatch | AuthTagError; emitted as a corrupted packet with no measurements |

The tag provides integrity and origin authentication only — the payload is not encrypted (FLAG_ENCRYPTED remains reserved). Replay is handled by sequence tracking: a sequence far behind the newest one is flagged replay = True and counted.

## Stream parsing

StreamParser.parse_stream(data: bytes) -> List[ParsedPacket] is resync-safe:

- It scans for the MAGIC sync word, so partial or noisy streams recover at the next valid frame boundary rather than desynchronising permanently.

- Each ParsedPacket exposes .header (dict), .payload (bytes), .crc_valid, .duplicate, .out_of_order, .replay, .auth_state, .rejected, and a .sequence property. StreamParser.counters() exposes all diagnostic counters.

- Duplicate and out-of-order frames are flagged, not silently dropped, so the quality accounting downstream stays honest.

## Forward Error Correction (optional)

When FLAG_FEC_PRESENT is set and the reedsolo package is installed, the pipeline’s FEC stage attempts Reed–Solomon recovery. If reedsolo is not installed, FEC is disabled explicitly and reported — corrupted frames are marked CORRUPTED rather than silently passed through as if valid.

## Decoding to measurements

decode_payload_to_frame(pkt) converts a ParsedPacket into a TelemetryFrame of fully-provenanced Measurement objects. Each measurement carries its value, unit, timestamp, sensor id, QualityState, DataSource, and uncertainty — never a bare number.

Payload limits (hostile input never raises): UTF-8 JSON object, ≤ 16 KiB, ≤ 64 fields, nesting depth ≤ 3, strings ≤ 64 chars. Non-finite or non-numeric values become INVALID; a declared source may only downgrade provenance to SIMULATED; implausible timestamps are replaced by receive time and flagged timestamp_replaced.