# AirOne — Quick Start Guide

Get the AirOne Ground Station running in under 5 minutes.

## Prerequisites

- Python 3.10, 3.11, or 3.12

- 2 GB RAM (4 GB recommended for ML features)

- 500 MB disk space

## Installation

# 1. Navigate to the application directory
cd airone

# 2. Install Python dependencies
pip install -r requirements.txt

# 3. (Optional) Install ML packages for full detector coverage
pip install scikit-learn torch

## Configuration

# Generate a secure JWT secret
export AIRONE_JWT_SECRET="$(python3 -c 'import secrets; print(secrets.token_urlsafe(48))')"

# Or set it manually (minimum 32 characters)
export AIRONE_JWT_SECRET="your_very_secure_secret_at_least_32_characters_long"

## Running the System

### Option 1: Live Mode (with real CanSat hardware)

Connect the ground LoRa E22 module (transparent mode, M0=M1=GND) to your laptop via a USB-serial bridge, then:

# Auto-discover the radio:
python3 launcher.py

# Or specify the port/baud explicitly (recommended for E22 transparent mode):
python3 launcher.py --serial-port /dev/ttyUSB0 --baud 115200 --force-baud

What happens: - System starts and binds API to http://localhost:5000 - Opens the E22 serial link (auto-discover, AIRONE_SERIAL_PORT, or --serial-port) and streams live telemetry through the real pipeline - Prints an explicit link state: LIVE (...), or NOT_CONFIGURED (...) when no radio is found — no data is ever fabricated - Logs to logs/airone.jsonl

Flight firmware: the matching ESP32 sketch lives in firmware/airone_cansat/. It emits the exact AirOne binary frames (verified byte-for-byte against the ground station). Full wiring and setup: docs/hardware_connection.md.

### Option 2: Simulation Mode (no hardware needed)

python3 launcher.py --simulate

What happens: - Generates synthetic CanSat telemetry (ISA atmosphere model) - Sends through full binary protocol + CRC - Processes through 20-stage pipeline - Runs all 37 scientific analyses - Perfect for testing, demos, and development

### Option 3: With GUI (requires PyQt5)

python3 launcher.py --gui --simulate

What happens: - Launches six-tab PyQt5 ground station interface - Mission Control, Science Dashboard, GNSS Map, Replay, AI Diagnostics, Config/System - Auto-refreshes with live telemetry

## Verify Installation

Run the full test suite:

pytest -v

Expected output: 154 passed in ~25s

## Access the API

Once running, the API is available at http://localhost:5000/api/v1/

### 0. Create the first administrator (once)

No accounts ship with the software:

AIRONE_ADMIN_PASSWORD='<strong password, 12+ chars>' python3 launcher.py --create-admin
# or: python3 launcher.py --create-admin --generate-password   (printed once)

### 1. Login to get JWT token

curl -X POST http://localhost:5000/api/v1/auth/login \
  -H "Content-Type: application/json" \
  -d '{"username": "admin", "password": "<the password you set>"}'

Response:

{
  "success": true,
  "data": {
    "access_token": "eyJ0eXAiOiJKV1QiLCJhbGc...",
    "refresh_token": "eyJ0eXAiOiJKV1QiLCJhbGc...",
    "user": {
      "username": "admin",
      "role": "admin"
    }
  }
}

### 2. Use the token for authenticated requests

# Save token
TOKEN="eyJ0eXAiOiJKV1QiLCJhbGc..."

# Get system health
curl http://localhost:5000/api/v1/system/health \
  -H "Authorization: Bearer $TOKEN"

# Get telemetry measurements
curl http://localhost:5000/api/v1/telemetry/measurements?limit=10 \
  -H "Authorization: Bearer $TOKEN"

# List scientific analyses
curl http://localhost:5000/api/v1/scientific/analyses \
  -H "Authorization: Bearer $TOKEN"

# Check ML detector availability
curl http://localhost:5000/api/v1/ml/detectors \
  -H "Authorization: Bearer $TOKEN"

## Docker Deployment (Production)

# 1. Set production JWT secret
export AIRONE_JWT_SECRET="$(python3 -c 'import secrets; print(secrets.token_urlsafe(48))')"

# 2. Start with Docker Compose (refuses to start without AIRONE_JWT_SECRET;
#    the API is published on 127.0.0.1:5000 only)
docker compose up -d

# 2b. Create the administrator inside the container (once)
AIRONE_ADMIN_PASSWORD='<strong password>' docker compose exec \
  -e AIRONE_ADMIN_PASSWORD airone python3 launcher.py --create-admin

# 3. Check logs
docker-compose logs -f airone

# 4. Access API
curl http://localhost:5000/api/v1/system/health

## Accounts

There are no default credentials. Create accounts with the launcher:

python3 launcher.py --create-user ops1 --role OPERATOR --generate-password
python3 launcher.py --reset-password ops1        # uses AIRONE_ADMIN_PASSWORD
python3 launcher.py --list-users

Roles: VIEWER < OPERATOR < SCIENTIST < ENGINEER < ADMIN. Admins can also manage accounts through POST /api/v1/users (see docs/api_reference.md).

For an isolated demo only, AIRONE_ALLOW_DEFAULT_USERS=1 seeds admin/AirOneAdmin!2026 and viewer/viewer123; both must change their password on first login (POST /api/v1/auth/change-password).

## File Locations

- Database: data/airone.db

- Logs: logs/airone.jsonl, logs/audit.jsonl

- Config: config/default_config.yaml

- Documentation: docs/ (13 comprehensive guides)

## Troubleshooting

### “InsecureConfigError: Refusing to start with default JWT secret”

Solution: Set AIRONE_JWT_SECRET environment variable (min 32 chars).

export AIRONE_JWT_SECRET="your_secure_secret_minimum_32_characters_long"

### “No module named ‘PyQt5’”

Solution: Either install PyQt5 or run headless:

# Option 1: Install PyQt5
pip install PyQt5

# Option 2: Run headless (no GUI)
export QT_QPA_PLATFORM=offscreen
python3 launcher.py --simulate

### “Serial port not found”

Solution: 1. Check device is connected: ls /dev/ttyUSB* (Linux) / Device Manager (Windows) 2. Add user to dialout group: sudo usermod -a -G dialout $USER (then re-login) 3. Specify the port explicitly: python3 launcher.py --serial-port /dev/ttyUSB0 (or export AIRONE_SERIAL_PORT=/dev/ttyUSB0) 4. Port opens but no packets? Baud mismatch — match the E22 UART baud: python3 launcher.py --serial-port /dev/ttyUSB0 --baud 115200 --force-baud 5. Full wiring, E22 configuration, and troubleshooting table: docs/hardware_connection.md

The launcher never fabricates data: with no radio it reports NOT_CONFIGURED and streams nothing (use --simulate for isolated testing).

### ML detectors show as “unavailable”

Solution: Install optional dependencies:

pip install scikit-learn torch

## Next Steps

- Read the documentation: docs/operator_manual.md for mission operations

- Explore the API: docs/api_reference.md for all endpoints

- Understand the science: docs/scientific_methods.md for all 37 analyses

- Review security: docs/security_model.md before production deployment

- Deploy to production: docs/deployment.md for Docker/systemd setup

## Quick Architecture Overview

CanSat (Serial) → TelemetryReceiver → PacketProcessor → Pipeline (20 stages)
                                            ↓
                    ┌───────────────────────┼───────────────────────┐
                    ▼                       ▼                       ▼
            ScientificWorker        MLAnalysisWorker      PersistenceWorker
                    │                       │                       │
                    └───────────────────────┴───────────────────────┘
                                            ↓
                                    SQLite Database
                                            ↓
                                    Flask REST API (port 5000)
                                            ↓
                                ┌───────────┴───────────┐
                                ▼                       ▼
                            PyQt5 GUI           External clients

## Support

For detailed troubleshooting, consult: - docs/troubleshooting.md — Common issues and solutions - docs/failure_modes.md — How each subsystem degrades - logs/airone.jsonl — Structured system logs

System is ready. Happy CanSat operations! 🚀